|  | Member of the MacTaggart Scott Group MACSCOTT BOND LTD Engineering with Hydraulics Aberdeen Division UNIT 2, BLACKNESS ROAD ALTENS INDUSTRIAL ESTATE ABERDEEN AB12 3LH Tel: 01224 873113 Fax: 01224 877700 |
| --- | --- |

###### OPERATING INSTRUCTIONS FOR ELECTRIC DRIVEN POWER UNIT SERIES 160-M

| Prior to use of this Equipment, these instructions should be Read , Understood and its Operation Carried out by Designated “Competent “ Personnel |  |
| --- | --- |
| NO: | INSTRUCTIONS: |
| 1 | Fill Tank. (Please ensure there is sufficient fluid in tank to cover the Float Switch) |
| 2 | Ensure Gate Valve is Open. |
| 3 | Connect to suitable Electrical Supply. |
| 4 | Loop Pressure Outlet to Return. |
| 5 | Open Pressure Outlet and Return Ball Valve. |
| 6 | Start pump and check flow on flowmeter 30 LPM on 50 HZ, 36 LPM on 60 HZ. Bleed Pump if Flow is low. |
| 7 | Back off Remote Relief Valve on panel. N.B. Do not adjust any other valve on unit. |
| 8 | Close Pressure outlet Ball Valve. |
| 9 | Increase Remote Relief Valve to required pressure. PRV set at 200Bar. |
| 10. | Stop Pump. |
| 11. | Connect to System. |
| 12. | Open Return and Pressure Ball Valves. |
| 13. | Start Pump. |
| ##### NOTE : Customers are advised to flush the Unit through with their own system fluid in order to eliminate the danger of system contamination due to unforseen ciscumstances and / or in the event of wrongly supplied fluid relating to fluid type. |  |