OPERATING INSTRUCTIONS FOR HYDRAULIC       FLUSHING UNIT    -     SERIAL NO:   MSB 060

- Fill Tank.

- Ensure Suction Ball Valve is Open.

- Connect to suitable Electrical Supply.

- Loop Pressure Outlet to Return.

- Open Pressure Outlet and Return Ball Valve.

- Start pump and check flow on flowmeter 25 LPM on 50 HZ,

30 LPM on 60 HZ.  Bleed Pump if Flow is low.  (See attached sheet.)

- Back off Remote Relief Valve on panel.

N.B  Do not adjust any other valve on unit.

8.	Close Pressure outlet Ball Valve.

- Increase Remote Relief Valve to required pressure.

N.B  2 Stage Pump,   Stage one unloads at 3000 PSI,  Stage two PRV set at 6200 PSI.

- Stop Pump.

- Connect to System.

- Open Return and Pressure Ball Valves.

- Start Pump.

## NOTE       Customers  are  advised to  flush the  Unit through  with  their own system  fluid

in order   to eliminate  the  danger of  system contamination  due to  unforseen

circumstances  and / or  in  the event  of wrongly supplied information relating

to the  fluid type.